NECHOL LTD

WORK ACTIVITY / RISK ASSESSMENTS

Popular Garage

113 Mill Street,

Leek,

Staffordshire Moorlands,

ST13 8EU.

Contents

Risk Assessment (Guide)

Generic Risk Assessments

Ref No                                                    Activity

G0                                                           Blank form

G1                                                           Ground worker – Excavations

G2                                                           Piling

G3                                                           Piling (auger)

G4                                                           Ground worker – Live sewers

G5                                                           Bricklaying – Ground level

G6                                                           Bricklaying – from Scaffold

G7                                                           Labourers Mixing and Loading

G8                                                           Cutting stonework at height

G9                                                           Carpenters – 1st & 2nd Fix

G10                                                         Carpenters – roof trusses

G11                                                         Carpenters – High level false work

G12                                                         Plaster & Dry lining

G13                                                         Painting – Domestic

G14                                                         Painting – Commercial

G15                                                         Painting – Industrial

G16                                                         Plumbing – Fitting out

G17                                                         Plumbing – Incomplete buildings

G18                                                         Glazier – Small panels by hand

G19                                                         Roofer – Tiling

G20                                                         Roofer – Hot bitumen/flat roof

G21                                                         Plant Operators – Dumper

G22                                                         Plant Operators – Excavators and Cranes

G23                                                         Demolition – Soft (non structural)

G24                                                         Demolition – General

G25                                                         Steel Erection

G26                                                         Scaffolding – Mobile Towers

G27                                                         Scaffolding – Independently

G28                                                         Scaffolding – High Slung

G29                                                         Stone Mason – Stone Cleaning

G30                                                         Asbestos

G31                                                         Petrol / Diesel Cement Mixer

G32                                                         Working at Height

RISK ASSESSMENTS

Before suitable Risk Assessments can be carried out with any degree of success, a basic understanding of

the requirements of what is expected by the legislation are needed.

The management of Health & Safety at Work Regulations 1992 require that risks are assessed.  Assessments

are only required on significant hazards, which could result in serious harm or effect several people.  There

is no requirement to carry out Risk Assessments on trivial matters within the workplace.

The Five Steps to Risk Assessment are:

- Take a fresh look at what could reasonably be expected to cause harm.  The workforce should

be able to offer suggestions on things that they may have noticed during the course of their work.

Other information will be available through sources such as manufacturer’s instructions or

data-sheets.  All the above will help to spot hazards and put the risks into perspective.

- On finding a significant hazard, decide who might be harmed and how they might be harmed.

Consideration must also be given to other persons who may not be in the workplace all the time, e.g. cleaners, maintenance personnel, members of the public and other persons/companies that may share the workplace.

- The risks that may arise from hazards that have been identified need to be evaluated.  Then a decision must be made whether the existing precautions are adequate or more are required.  Can the hazard be got rid of altogether, if not can the risks be controlled so that harm is unlikely?

It must be remembered that even when all the systems and precautions have been put into place some risk may remain.

Questions that you may need to ask are:

- Have you done all that the law requires?

- Are generally accepted industry standards in place?

c)   Have you done everything that is reasonably practicable?

- If you employ more than five persons you must record the significant findings of your assessments.

There is no requirement to show how the assessments were carried out providing you can show:

a)   That a proper check was made.

b)   You asked who might be affected.

c)   You dealt with all the obvious significant hazards, taking into account the number of people who

could be involved.

d)   The precautions are reasonable and the remaining risk is low.

It is not expected that assessments are perfect but suitable and sufficient. The two key points are:

1)   Are the precautions reasonable?

2)   Is information available to show a proper check was made?

5.   Review your assessments from time to time and revise as necessary.  The assessments do not need to

be changed for trivial changes but they will need to be changed for significant changes that may

increase the hazard or the risk.

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G0 |  |  |
| --- | --- | --- |
| Method |  |  |
| Hazards |  |  |
| Risks |  |  |
| Population Exposed |  |  |
| Controls Statutory |  |  |
| Other Controls |  |  |
| Consultation |  |  |
| Documentation |  |  |
| Implementation |  |  |
| Information Instruction |  |  |
| Training |  |  |
| PPE |  |  |
| Monitoring |  |  |
| Systems Review |  |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G1 | Ground Worker - Excavations | MEDIUM RISK |
| --- | --- | --- |
| Method | General work within excavations, including bottoming out. Deep excavations will require specific assessment. |  |
| Hazards | Site activity and conditions: Access/Egress: Collapse of excavation, Gases and lack of Oxygen, Head and Foot. |  |
| Risks | Crushing injury from collapse, falling into excavation, poisoning/asphyxiation, Head and Foot injury. |  |
| Population Exposed | Operatives primarily. |  |
| Controls Statutory | Access – Excavations above 1.2m adequately supported (unless battered back sufficiently), inspected on a daily basis, thorough examination weekly (enter in register) or if fall of earth. Spoil to be stored back from edge. |  |
| Other Controls | Excavating should be closely supervised by an experienced, competent person. Adequate supplies of timber/sheeting available and inspected before use. Excavations to be preceded by check (drawings etc.) for buried services, and use of detection device. Boundaries of excavation to be guarded, and vehicle and other plant operators advised of safe working distances. Suitable Access/Egress arrangements including ladders etc, required. Where Gas hazards (or lack of oxygen) possible – system for gas monitoring, ventilation times etc, managed by detailed permit to enter, Based on COSHH assessment of controls required. Feet – site is foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | Safety plan for excavations required. |  |
| Implementation | Supervisor to ensure safety plan adhered to and inspect excavations daily. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks/precautions for work in excavations. |  |
| Training | Ensure operatives receive training on safety in excavations. |  |
| PPE | Head and Foot protection. Protective clothing in wet conditions. |  |
| Monitoring | Supervisor to periodically check arrangements during progress of job. Gas/Oxygen monitoring is required. |  |
| Systems Review | Review periodically to confirm validity of safety plan. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G2 | Piling – Drop piling on contaminated site near water | HIGH RISK |
| --- | --- | --- |
| Method | Hammering piles into ground to stabilise it. |  |
| Hazards | Site; Noise, Soil Contaminants, Water Contaminants, Water, erection and dismantling, Foot and Head. |  |
| Risks | Fall of persons/equipment into water, exposure to harmful contaminants causing ill health Hearing loss, erection and dismantling, Foot and Head. |  |
| Population Exposed | Operatives primarily. Local residents – noise (nuisance). |  |
| Controls Statutory | Driver train equipment may include craneage subject to Regulations. Control of Pollution Act may apply. In prohibiting noise nuisance to site neighbours (noise levels/working hours). Soil and water. Contaminants subject to COSHH. Head protection. |  |
| Other Controls | Work plan to ensure operatives at minimal risk of injury from equipment – utilising competent, skilled staff. Work area adjacent to water to Be checked by competent engineers to identify safe working positions – to be clearly set out and marked. Water hazards to be clearly signed, life preserves, life belts, safety boats, whistles etc, to be issued and located as required. Hydraulic pilling to be considered, but if not practicable then noise hazard to be marked, and all operatives to wear hearing defenders. Method Statement for safe erection and dismantling. Soil and water contaminants (Weil’s disease) to be individually evaluated to establish precautions required……………………. (COSHH Assess). Feet – site is foot protection zone safety boots to be worn. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | Detailed work Plan/Method Statement required. Method statement to include erection/dismantling rig. |  |
| Implementation | Supervisor to ensure work equipment and arrangements satisfactory. Hygiene arrangements and Welfare facilities for operatives in place. |  |
| Information Instruction | New employees – induction on company policy and procedures, and safe working before permitted to work. Briefing on risk/precautions for piling – injury from equipment, noise. Working adjacent to water and working on contaminated land – briefing. Guidance on health surveillance, if required, together with use of site issue sheet detailing contamination for use with personal GP if illness arises. |  |
| Training | Supervisors/Managers require training in statutory responsibilities, project planning, establishment and running of site organisation. Ensure operatives receive specific training on use of equipment. Training essential concerning noise, soil and water contaminants and precautions required. G2 Continued……. |  |
| PPE | Head protection and hearing defenders (subject to noise assessment). Foot protection to be worn. Other protective equipment and clothing to be dependent on COSHH Assessment of contaminants. |  |
| Monitoring | Depending upon contaminants, may require monitoring of airborne material. Usage of hygiene (washing) facilities etc. May require noise measurements. Constant supervision to check arrangements during progress of job. |  |
| Systems Review | Review throughout works to confirm validity of work Plan/Method Statements. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G3 | Piling – augur piling | MEDIUM RISK |
| --- | --- | --- |
| Method | Setting up, Augur removal of materials to facilities creation of ground stabilising piles, dismantling. |  |
| Hazards | Site activity and conditions: Noise, Contact with moving machinery, erection and dismantling, Foot and Head. |  |
| Risks | Noise Induced Hearing Loss, Injuries from moving machinery, Foot and Head. |  |
| Population Exposed | Operatives primarily. Local residents – noise (nuisance). |  |
| Controls Statutory | Drive train equipment may include craneage subject to Regulations. Control of Pollution Act may apply in prohibiting noise nuisance to site neighbours (noise levels and working hours). Head protection. |  |
| Other Controls | Work plan to ensure operatives at minimal risk of injury from equipment – utilising competent, skilled staff. Ground conditions to be checked to ensure soil contaminants absent (augur may spray mud/water when clearing between downward drives – for contaminated ground specific assessment required). Feet – site is foot protection zone, safety boots to be worn. Method Statement for safe erection and dismantling. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | Work plan for piling, Method Statement for erection/dismantling rig. |  |
| Implementation | Supervisor to ensure work equipment and arrangement satisfactory. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks/precautions for piling – injury from equipment, noise. |  |
| Training | Ensure equipment operatives receive specific training on use of equipment. |  |
| PPE | Head protection. May require hearing defenders (subject to noise assessment) arising from noise emissions of drive train. Foot protection to be worn. In wet conditions (below water table) protective clothing and waterproof boots require to protect operatives from mud/water spray. |  |
| Monitoring | May require noise measurements. Supervisor to periodically check arrangements during progress of job. |  |
| Systems Review | Review periodically to confirm validity of Work Plan. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G4 | Groundworker – breaking into live sewer | HIGH RISK |
| --- | --- | --- |
| Method | General work within excavations, including bottoming out. Work involves breaking into existing sewer. |  |
| Hazards | Site activity and conditions; Access/Egress; Collapse of excavations, Gas, Lack of oxygen and Micro biological hazards, Head and Foot. |  |
| Risks | Crushing injury from collapse, falling into excavation, Explosion/Fire, Infection, Gas poisoning/asphyxiation, Head and Foot. |  |
| Population Exposed | Operatives primarily, other adjacent workers. |  |
| Controls Statutory | Access – Excavations above 1.2m adequately supported (unless battered back sufficiently), inspected on a daily basis, through examination weekly (Enter in Register) of if fall of earth. Spoil to be stored back from edge. |  |
| Other Controls | Liaison with utility providing sewerage. Detailed work plan to divert/contain flow should storm etc, increase groundwater. System for detecting explosive gases (monitoring device). Permit to work controlled. Operatives protected against contamination (clothing) and provided with good hygiene facilities, waterproof plasters etc. No hot work/naked flames/smoking. (COSHH Assess) Excavating should by closely supervised by an experienced, competent person. Adequate supplies of timber/sheeting available and inspected before use. Excavations to be preceded by check (drawings etc) for buried services, and use of plant operators advised of safe working distances. Feet – site is foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | Safety plan for excavations to be developed into permit to work for live sewer work. |  |
| Implementation | Supervisor to ensure safety plan adhered to, and inspect excavations daily. Supervisor responsible for signing Permit, including checks that gas monitor functioning correctly. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks/precautions for work in excavations, and live sewer work. |  |
| Training | Ensure operatives receive training on safety in excavations, risk of Weil’s Disease (Leptospira) and gas explosion/fire. |  |
| PPE | Head protection. Protective clothing and boots. |  |
| Monitoring | Constant supervision to regularly check arrangements during progress of job. Gas monitoring for methane/oxygen levels. |  |
| Systems Review | Review periodically to confirm validity of safety plan and Permit to work system. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G5 | Bricklaying/Block laying off slab or at ground level | LOW RISK |
| --- | --- | --- |
| Method | Trowelling mixed mortar, manual handling of bricks/blocks, pointing. |  |
| Hazards | Site activity and conditions; manual handling, wet mortar, Head (general site), Foot. |  |
| Risks | Back strain (large blocks), repetitive strain, skin problems, Foot and Head injury. |  |
| Population Exposed | Operatives primarily. |  |
| Controls Statutory | HSE Guidance on Building Blocks (sheet 37) interpreting Manual Handling Regulations. If lifting devices used to move blocks, require inspections and records. |  |
| Other Controls | 20kg maximum weight for repetitious laying of blocks by lone worker. Above 20kg mechanical aids of team work. Attempt to secure client approval for lighter blocks to be specified (double skinning if necessary). Work planned to minimise handling, storing materials close to hand, raising level of storage to minimise lift, providing good access, minimising turning movements. Mechanical aids or team work specified for heavy blocks. Mortar and skin. Feet – site is foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with client and employees. |  |
| Documentation | Method for handling heavy blocks to be documented if necessary. |  |
| Implementation | Supervisor to ensure work area property set up to minimise handling problems. Good hand washing facilities and gloves to be available. Supervisor/Operatives to check that access to working areas is safe. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. |  |
| Training | Ensure operatives receive kinetic handling briefing and training in use of mechanical aids. |  |
| PPE | Head and Foot protection to be worn. Gloves to be available if required. |  |
| Monitoring | Supervisor to occasionally check manual handling arrangements during progress of job. |  |
| Systems Review | Review periodically to confirm back strain injuries minimised. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G6 | Bricklaying – heavy building blocks off scaffold | HIGH RISK |
| --- | --- | --- |
| Method | Trowelling mixed mortar, manual handling of bricks/blocks, pointing. |  |
| Hazards | Site activity and conditions; manual handling, wet mortar, Head and Foot injury. |  |
| Risks | Back strain, repetitive strain, skin problems, Foot and Head injury. |  |
| Population Exposed | Operatives primarily. Worker below (fall of materials). |  |
| Controls Statutory | Scaffold with Brick Guards, inspections and entry in Register. HSE Guidance on Building Blocks (sheet 37) interpreting Manual Handling Regulations. If lifting devices used to move blocks, require inspections and records. Head protection. |  |
| Other Controls | 20kg maximum weight for repetitious laying of blocks by lone worker. Above 20kg mechanical aids of team work. Attempt to secure client approval for lighter blocks to be specified (double skinning if necessary). Work planned to minimise handling, storing materials close to hand, raise storage areas to minimise lift, providing good access, minimising turning movements. Mechanical aids or team work required for moving/laying the blocks. Mortar and skin. (COSHH Assess) Feet – site is foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | Method for handling heavy blocks to be documented. |  |
| Implementation | Supervisor to ensure work area property set up to minimise handling problems. Good hand washing facilities and gloves to be available. Supervisor/Operatives to check that access to working areas is safe. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. |  |
| Training | Ensure operatives receive kinetic handling. |  |
| PPE | Head and Foot protection to be worn. Gloves to be available if required. Head and Foot protection to be worn. Gloves to be available if required. |  |
| Monitoring | Constant supervision to regularly Supervisor to occasionally check manual handling arrangements during progress of job. |  |
| Systems Review | Review at end of each major contract to confirm back strain injuries minimised. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G7 | Labourers Mixing and Loading Out for a Bricklayer | LOW RISK |
| --- | --- | --- |
| Method | Loading Out, Mixing (Hand and Machine), Transporting, Gaining Access, Unloading, Combination hand work and use of limited mechanical aids. |  |
| Hazards | Site activity and site conditions; Manual handling; Dust and wet mortar – lungs and skin; Mechanical and electrical equipment; Transport – manual and mobile plant; Access, falls, Transport and Unloading. |  |
| Risks | Back strain; Falls (material operative); Mechanical injury (equipment and transport); Electric shock; Respiratory tract (dust), Eyes and Skin; Foot injury. |  |
| Population Exposed | Operatives primarily. Fellow employees (mobile plant, fall materials) Occupiers if adjacent. Members of the public (if at site boundary). |  |
| Controls Statutory | Scaffold/Working Platforms. Electrical, Head Protection. Access equipment inspections – entries in Register. |  |
| Other Controls | Dust – advise work upwind. Skin – gloves, overalls outside boots, hand washing. Feet – site is foot protection zone, safety boots to be worn. Mech.Equip and Electrical – daily check by operative, monthly recorded check by fitter. Transport – traffic segregated from work areas by physical barriers. Separate access provided for occupiers. Manual handling – hoist in use, but significant manual handling requires documented procedures and operative to receive kinetic handling training. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | Above to be reinforced in brief method Statement. |  |
| Implementation | Supervisor to check at commencement that above arrangement in force. |  |
| Information Instruction | New employees – induction on company policy and procedure and for safe site working before permitted to work. All – toolbox talk on Method Statement before work starts. Maintenance Staff – briefing on maintenance and electrical checks. |  |
| Training | Ensure maintenance staff have required competency. Ensure operatives have been trained in use of mobile (transport) equipment and hoist. Ensure access equipment inspected by competent persons. Ensure supervisor trained in general safety and in skin checks. |  |
| PPE | Hard Hat; Safety Boots; Gloves; Overalls. G7 continued………….. |  |
| Monitoring | Supervisor to occasionally recheck arrangements during progress of job. Monthly skin inspection (health surveillance). Site manager to carry out inspections and audits periodically – this low risk task does not determine audit period. |  |
| Systems Review | Project not individual task. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G8 | Cutting out and replacing Stonework at High Level | HIGH RISK |
| --- | --- | --- |
| Method | Cutting out and removing existing stonework using hand/power tools, lifting equipment, replacing with new stonework and bedding into place. |  |
| Hazards | Site activity and conditions; Access, Falls of persons/materials; Manual handling; Equipment: Dust – silica; Contaminants – Bird droppings etc. Noise; Vibrations Head, Feet and Eyes; Mastics and Bedding compounds. |  |
| Risks | Back strain; Electric shock, Mechanical injury; Lungs: Vibration White Finger; Eye and Foot injury; Contamination from bird droppings etc, Falls (persons/materials). |  |
| Population Exposed | Operatives primarily. Other workers, occupiers, members of public (adjacent to or below). |  |
| Controls Statutory | Scaffold/Working Platform/Ladder standards and other aspects of safe access. Scaffold inspections and entries in Register. Electrical checks on equipment, low voltage to be used on site. Inspections of lifting equipment and certification. Head protection. |  |
| Other Controls | Dust – silica and bird droppings, moss, algae, lichen – RPE (COSHH Assess). Dust collection to minimise airborne, use of damping down, etc. Contamination – personal hygiene (hand washing, no smoking etc) (COSHH Assess). Mastics and bedding compounds – minimise skin contact (COSHH Assess). Vibration – vibration – absorbing handles on cutting equipment, gloves if cold. Noise assess to decide on hearing protection (Noise Assessment). Feet – site foot protection zone, safety boots supplied and must be worn. Manual handling – operative to receive kinetic handling training. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees and occupiers. |  |
| Documentation | Work requires a Method Statement. Access equipment inspections – entries in Register. |  |
| Implementation | Supervisor to check at commencement that above arrangements in force. |  |
| Information Instruction | New employees – inducted on company safety policy and procedures, and for safe working before permitted to work. Personal protection against airborne dust (silica, bird lime etc) and vibration. |  |
| Training | Ensure operatives receive kinetic handling refresher, and training use of plant and equipment. |  |
| PPE | Hard Hat; Safety Boots; RPE, Gloves, Overalls. |  |
| Monitoring | Constant supervision to recheck arrangements during progress of job. |  |
| Systems Review | Review of Method Statement to ensure applicability (with necessary modifications for local conditions) on other sites. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G9 | Carpenters – First and Second Fixing | LOW RISK |
| --- | --- | --- |
| Method | General carpentry/joinery – fixing doors etc. during second fixing with hand tools and few portables, using hop-ups and other low level access. |  |
| Hazards | Site activity and conditions; Access; Manual handling; Equipment; Dust and adhesives; Head, Feet and Eyes. |  |
| Risks | Back strain; Electric shock, mechanical injury; Head, Eye and Foot injury; Skin and Lung exposed to dust and adhesives. |  |
| Population Exposed | Operatives primarily. |  |
| Controls Statutory | Scaffold/Working Platforms/Ladder standards and other aspects of safe access Electrical checks on equipment, low voltage used on site. Head protection (for general site conditions as necessary). |  |
| Other Controls | Dust and adhesives – avoid exposure, good hygiene. If using tools which create eye risk – wear protective goggles. Feet – site is foot protection zone, safety boots to be worn. Manual handling – load out by machine close to point of use, use of mechanical aids (trolleys etc), operative to receive kinetic handling training. Tools to be checked by tradesmen – faults to be reported, equipment repaired. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | No special documentation required. |  |
| Implementation | Tradesmen competent – required to advise if there are problems. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. |  |
| Training | Ensure operatives receive kinetic handling refresher, and training in use of plant and equipment. |  |
| PPE | Hard Hat, Safety Boots, Goggles (if required). |  |
| Monitoring | Supervisor to occasionally check arrangements during progress of job. |  |
| Systems Review | Project not individual task. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G10 | Carpenters – Installing Roof Trusses | MEDIUM RISK |
| --- | --- | --- |
| Method | Raising trusses by machine, installation, high level access. |  |
| Hazards | Site activity and conditions; Access; Manual handling; Equipment; Pre-treated timber; Head, Feet and Eyes. |  |
| Risks | Falls from height; Back strain; Electric shock, mechanical injury; Head, Eye and Foot injury; Skin exposed to wet treatment chemicals (if timber uncured), hand injury (sawn timber). |  |
| Population Exposed | Operatives primarily, others working below. |  |
| Controls Statutory | Access including temporary platforms – Inspections of scaffolds and entries in Register. Electrical checks on equipment, low voltage used on site. Lifting equipment inspections and certification. Head protection. |  |
| Other Controls | Avoid handling wet (uncured) treated timber. (COSHH Assess) If using tools which create eye risk – wear protective goggles. Feet – site is foot protection zone, safety boots to be worn. Manual handling – use machines as far as possible, operatives to receive kinetic handling training. Tools to be checked by tradesmen – faults to be reported, equipment repaired. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | Brief Method Statement an advantage. |  |
| Implementation | Supervisor to check that access is safe, lifting equipment in order, etc. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. |  |
| Training | Ensure operatives receive kinetic handling refresher, and training in use of plant and equipment. Ensure operatives have been trained in use of lifting equipment. Ensure access equipment inspected by competent (trained) persons. |  |
| PPE | Hard Hat, Safety Boots, Gloves |  |
| Monitoring | Supervisor to occasionally check arrangements during progress of job. |  |
| Systems Review | Project not individual task. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G11 | Carpenters – High Level Falsework | HIGH RISK |
| --- | --- | --- |
| Method | Forming Falsework, high level access, hand tools and portable electrical equipment. |  |
| Hazards | Site activity and conditions; Access – falls; Manual handling; Equipment; Head, Feet and Eyes; Hand injury (rough cut timber). |  |
| Risks | Falls from height; Back strain; Electric shock, mechanical injury; Head, Eye, Hand and Foot injury. |  |
| Population Exposed | Operative primarily, others working below. Occupiers/Members of public if at site boundary. |  |
| Controls Statutory | Access – inspections of scaffold/Working Platforms and entries in Register. Electrical checks on equipment, low voltage used on site. Lifting equipment inspections and certification. Head protection. |  |
| Other Controls | For edge work, use harnesses and/or other safety equipment. Design of Falsework – stability, buildability. Access – quality of scaffolding, provision of ladders etc to ensure good access. Feet – site is foot protection zone, safety boots to be worn. Manual handling – load out by machine close to point of use, use of mechanical aids, operatives to receive kinetic handling training. Tools to be checked by tradesmen – faults to be reported and equipment repaired. |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | Detailed Method Statement required. |  |
| Implementation | Supervisor to check that access is safe, lifting equipment in order, etc. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. All – briefing on Method Statement before work commences. |  |
| Training | Ensure operatives receive kinetic handling refresher, and training in use of plant and equipment. Ensure operatives have been trained in use of lifting equipment. Ensure access equipment inspected by competent (trained) persons. |  |
| PPE | Hard Hats, Safety Boots, Gloves for timber handling if required. Harnesses, Belts, nets etc where necessary. |  |
| Monitoring | Constant Supervision to check arrangements during progress of job. |  |
| Systems Review | Review each job on completion to confirm validity of general approach. Falsework a regular candidate for auditing. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G12 | Plasters/Dry liner | LOW RISK |
| --- | --- | --- |
| Method | Sand/cement screed, plastering and drylining walls inside premises. |  |
| Hazards | Site activity and conditions; Access; Manual handling; Head, Feet and Eyes; Handling dry and wet materials and jointing compounds; Portable appliances. Hand injuries from studding systems and other materials. |  |
| Risks | Falls from height; Back strain; Electric shock; Head, Eye, Hand and Foot injury. |  |
| Population Exposed | Operatives primarily. |  |
| Controls Statutory | Scaffolds/Working Platforms where used, inspected and entry in Register. Safe use of ladders, hop ups etc. Electrical checks on equipment, low voltage to be used on site. Head protection (for general site conditions as necessary). |  |
| Other Controls | Feet – site is foot protection zone, safety boots to be worn. Manual handling; sheet materials – load out by machine close to point of use, use of mechanical aids, operative to receive kinetic handling training. Bags of materials – consider smaller bags. Consider use of PPE (gloves) when handling metal sections or sawn timber studding. Dry and wet materials, jointing compounds – avoid contact, personal hygiene. (COSHH Assess) |  |
| Consultation | Above to be discussed at a brief pre-work meeting with employees. |  |
| Documentation | No special documentation required. |  |
| Implementation | Supervisor/operatives to check that access to working area is safe. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. |  |
| Training | Ensure operatives receive kinetic handling briefing. |  |
| PPE | Hard Hat if required, Gloves if necessary (drylining). |  |
| Monitoring | Supervisor to occasionally check arrangements during progress of job. |  |
| Systems Review | Review annually to confirm validity of general approach. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G13 | Painting - Domestic | LOW RISK |
| --- | --- | --- |
| Method | Brush and roller application of range of paints from low level access equipment (ladders trestles etc) |  |
| Hazards | Site activity and conditions; Access; Handling wet paint; Solvents (oil based) |  |
| Risks | Falls from height; Skin contact; Solvent Vapour (oil based) |  |
| Population Exposed | Operative, 3rd parties/operatives passing below. Occupiers – solvent vapour. |  |
| Controls Statutory | Scaffolds/Working platforms were used, inspected and entry in Register. Head protection (for general site conditions as necessary). |  |
| Other Controls | Safety in use of ladders, trestles etc – check before use etc. Use non-oil paints wherever possible. For oil based, ensure good ventilation (plan for ventilation equipment if necessary). Consider demarcation/protection of 3rd parties. Paint – avoid contact, good personal hygiene, and provision of hand cleaning material. (COSHH Assess) |  |
| Consultation | Above to be discussed at brief pre-work meeting with employees. |  |
| Documentation | No special documentation required. |  |
| Implementation | Supervisor to ensure access equipment standard, general precautions implemented. Operatives to check that access to working areas is safe including towers. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. |  |
| Training | Ensure operatives (including apprentices) receive briefing on safe access and hand cleaning, etc. |  |
| PPE | Safety footwear may be required if other construction site activities create foot injury risks. Head protection may, similarly be required. |  |
| Monitoring | Supervisor to occasionally check access arrangements during progress of job. |  |
| Systems Review | Review annually to confirm validity of general approach. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G14 | Painting - Commercial | MEDIUM RISK |
| --- | --- | --- |
| Method | Brush and roller application of eggshell or other solvent based paints from low level access equipment (ladders, trestles etc). |  |
| Hazards | Site activity and conditions; Access; Handling wet paint; Solvents. |  |
| Risks | Falls from height; Skin contact; Solvent Vapour. |  |
| Population Exposed | Operative primarily. Operatives/3rd parties passing below. Occupiers – solvent vapour – levels well in excess of Maximum Exposure limit may easily be generated. |  |
| Controls Statutory | Scaffolds/Working platforms where used, inspected and entry in Register. Head protection (for general site conditions as necessary) |  |
| Other Controls | Safety in use of ladders – check before use etc. Attempt to secure client agreement for substitution of acrylic or other non-solvent based paint. If not possible, area isolated/sealed off, forced ventilation, respiratory protection and exclusion of non essential personnel from area. Paint – avoid contact, good hygiene, and provision of hand cleaning material. (COSHH Assess) |  |
| Consultation | Above to be discussed at a brief pre-work meeting with occupier and employees. |  |
| Documentation | If oil-based used – Method Statement and COSHH Assessment required. |  |
| Implementation | Supervisor to ensure access equipment is to the required standard, general precautions implemented including ventilation, and RPE is suitable and in use. Operatives to check that access to working area is safe including towers. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risk/precautions for large-scale use of solvent eggshell. |  |
| Training | Ensure operatives receive training in use of PPE and briefing on kinetic handling, safe access, and hand cleaning. |  |
| PPE | May require air-fed respirators; If ventilation good, possible that charcoal filtered respirators adequate. |  |
| Monitoring | May require air testing to measure airborne solvent. Supervisor to periodically check arrangements during progress of job. |  |
| Systems Review | Review periodically to confirm validity of Method Statement and preventative arrangements. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G15 | Painting - Industrial | HIGH RISK |  |
| --- | --- | --- | --- |
| Method | Mix and spray application of two-pack isocyanate or similar paint to high level steelwork from mobile (tower) scaffolds. |  |  |
| Hazards | Site activity and conditions; Access; handling wet paint; Isocyanates (resin). |  |  |
| Risks | Falls from height; Skin contact; Airborne isocyanate, head injury. |  |  |
| Population Exposed | Operative primarily. Occupiers – isocyanate aerosol. |  |  |
| Controls Statutory | Scaffolds/Working platforms where used, inspected and entry in Register. Head protection. |  |  |
| Other Controls | Exclude other workers/3rd parties from area – lung protection zone. Isolate and seal off area where possible. Avoid inhalation and skin/eye contact, good hygiene, provision of hand cleaning material. (COSHH Assess) |  |  |
| Consultation | Above to be discussed at a detailed pre-work meeting with employees. |  |  |
| Documentation | Due to toxicity of paint, Method Statement advisable. |  |  |
| Implementation | Supervisor to ensure access equipment to required standard, general precautions implemented and RPE is suitable in use. Operatives to check that access to working areas is safe including towers. |  |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Warning that exposure to Isocyanates may cause asthma. |  |  |
| Training | Ensure operatives receive training in use of spray equipment, and on precautions required for spray painting, erection and dismantling of towers and correct use of RPE. |  |  |
| PPE | Air fed respiratory protection (note: check air quality from compressor). Skin and eye protection – clothing, gloves, face visor. Head protection. |  |  |
| Monitoring | Constant supervision to check access and personal protection arrangements during progress of job. Note: Health surveillance required for operatives (isocyanates and asthma) |  |  |
| Systems Review | Review annually to confirm validity of general approach. |  |  |
| WORK ACTIVITY ASSESSMENT |  |  |  |

| Work Activity Ref G16 | Plumbing – Fitting out (second fitting) | LOW RISK |
| --- | --- | --- |
| Method | Provision of preliminary fixing, cold and hot work to form connections, thread pipes etc and install. (Separate assessment required for pressure testing) |  |
| Hazards | Site activity and conditions; manual handling, dust and noise (fixing), falls, mechanical equipment, contact with lubricants/sealants etc. Head (site conditions) and Feet. |  |
| Risks | Accidents arising from site conditions, manual handling injury from pipe threading machine, skin/eye contact with materials, fumes from hot work. Head injury (site conditions), Foot injury, Noise (drilling/fixing) |  |
| Population Exposed | Operative primarily. |  |
| Controls Statutory | Scaffolds/Working platforms where used, inspected and entry in Register. Head protection. Machinery guarding (e.g. pipe threading machines) |  |
| Other Controls | Work areas to be inspected for safe access prior to work commencing. Heavy pipes (4” and above) to be carried mechanically or by operatives working in pairs. Pipe threading machine to be set up in suitable area, guarded and fitted with effective foot control. Materials/Fumes (hot work), lubricants etc. Fire precautions and permit to work (hot work). Feet – site is foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at brief pre-work meting with employees (Occupiers where necessary for Hot work etc). |  |
| Documentation | Hot work permit may be required. |  |
| Implementation | Supervisor/Operatives to check that access to working area is safe, that tools and equipment are in good order. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on selection and use of fixing and anchors to ensure capable of withstanding typical loading. New employees – induction on company policy and procedures and for safe working before permitted to work. |  |
| Training | Ensure operatives receive kinetic handling briefing for large diameter pipe work. Tradesman skills required for work – by training and/or experience |  |
| PPE | May require PPE for hot work, and to protect hands against sealants etc. Gloves should not be worn whilst operating pipe-threading machine. Head, feet and hearing protection as necessary. |  |
| Monitoring | Supervisor to check arrangements at commencement of job. Work area inspected (hot work) 1 hour after work ceases and before leaving site. |  |
| Systems Review | Review periodically to confirm safety of site set up. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G17 | Plumbing Carcassing – incomplete premises | MEDIUM RISK |
| --- | --- | --- |
| Method | Provision of preliminary fixing, cold and hot work to form connections, thread pipes etc and install in incomplete premises. (Separate assessment required for pressure testing) |  |
| Hazards | Site activity and conditions; manual handling, dust and noise (fixing), falls, mechanical equipment, contact with lubricants/sealants etc. Head and Feet. |  |
| Risks | Risk of accidents arising from site conditions – Falls from height, but also manual handling injury from pipe threading machine, skin/eye contact with materials, fumes from hot work. Head and Foot injury. |  |
| Population Exposed | Operative primarily. |  |
| Controls Statutory | Scaffolds/Working platforms where used, inspected and entry in Register. Care should be taken in areas where slabs are incomplete and there are voids, holes etc requiring fencing. Head protection (for general site conditions as necessary). Machinery guarding (e.g. pipe threading machines). |  |
| Other Controls | Work areas to be inspected for safe access prior to work commencing. Heavy pipes (4” and above) to be carried mechanically or by operatives working in pairs. Pipe threading machine to be set up in suitable area, guarded and fitted with effective foot control. Materials/Fumes (hot work), lubricants etc. Fire precautions and permit to work (hot work). Feet – site is foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at brief pre-work meting with employees |  |
| Documentation | No special documentation required. |  |
| Implementation | Supervisor/Operatives to check that access to working area is safe, that tools and equipment are in good order. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on selection and use of fixing and anchors to ensure capable of withstanding typical loading. |  |
| Training | Ensure operatives receive kinetic handling briefing for large diameter pipe work. Tradesman skills required for work – by training and/or experience. |  |
| PPE | May require PPE for hot work, and to protect hands against sealants etc. Gloves should not be worn whilst operating pipe threading machine. Head, feet and hearing protection as necessary. |  |
| Monitoring | Supervisor to check arrangements at commencement of job, and periodically check access arrangements to minimise risk of falls from height. |  |
| Systems Review | Review periodically to confirm safety of site set up. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G18 | Glazier – Small panels by hand | LOW RISK |
| --- | --- | --- |
| Method | Cutting out existing glass, measuring and cutting glass panels by hand, fixing with proprietary putties/mastic. |  |
| Hazards | Site activity and conditions; Access, glass. |  |
| Risks | Hand/eye injuries cleaning frame and cutting/handling glass, head (general site conditions), foot. |  |
| Population Exposed | Operative primarily. 3rd parties from waste glass. |  |
| Controls Statutory | Access – check safe access (ensure that panel may be fitted from inside structure if possible). |  |
| Other Controls | Protection of hands and eyes during cutting out and glass handling by good working methods and use of PPE if required. Disposal of waste glass to avoid risk to others. Glazing to be undertaken off scaffolding where possible. |  |
| Consultation | Above to be discussed at pre-work meeting with employees. |  |
| Documentation | No special documentation required. |  |
| Implementation | Supervisor/Operatives to check that access to working areas are safe. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. |  |
| Training | General training as competent glazier, to handle, cut and fit glass safely and securely. |  |
| PPE | Hand/forearm protection usually required (loading out), eye protection may be required during cutting out. Head and foot protection as necessary. |  |
| Monitoring | Supervisor to occasionally check that operatives following arrangements discussed during induction. |  |
| Systems Review | Review periodically to confirm low risk nature of work. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G19 | Roofer – Tiling on domestic roof | MEDIUM RISK |
| --- | --- | --- |
| Method | Climbing onto roof to replace/fit clay tiles with pin fixings to battens already laid, cutting tiles with machines. |  |
| Hazards | Site activity and conditions; Access; manual handling of tiles, Head, Dust, Noise. |  |
| Risks | Falls from height, Head injury (working below others), Lungs, Hearing loss. |  |
| Population Exposed | Operative primarily. Others working/passing below. |  |
| Controls Statutory | Scaffolds/Working platforms where used, inspected and entry in Register. Edge protection required for jobs over two meters. And not of short duration. Electrical equipment – low voltage to be used. |  |
| Other Controls | For works greater than patch repairs taking longer than short duration, access to include for fixed scaffold to statutory standards and edge protection. For movement up roof, roof ladders should be secured from fixed structure/roped from far side (not ridge tiles). Arrangement for taking tiles to roof and (if required) disposing of old tiles – including chute to skip. Work to cease if adverse weather – snow, ice, high wind etc. Head protection (for working below) and foot protection (mid-sole protection not advisable for roof working) |  |
| Consultation | Above to be discussed at a detailed pre-work meeting with employees. |  |
| Documentation | Access arrangements to be in a standard form. |  |
| Implementation | Supervisor and operatives to check that access to working areas is safe, ladders are suitable, and edge protection is in place. |  |
| Information Instruction | Operative primarily. Briefing on risks/precautions for roof work. |  |
| Training | Ensure operatives receive basic training on safety in roof work, and in safely manually handling tiles. |  |
| PPE | Suitable footwear (for grip). Gloves may be used for handling tiles. Note that if machines used to cut tiles, eye and ear protection (and RPE) should be worn. Head protection if working below (e.g. passing up materials). |  |
| Monitoring | Supervisor to periodically check arrangements during progress of job, and take particular care to maintain review of inexperienced workers. |  |
| Systems Review | Review periodically to confirm validity and efficacy of safety arrangements. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G20 | Roofer – Hot bitumen application to flat roof | HIGH RISK |
| --- | --- | --- |
| Method | Heating of bitumen at ground level, hoisting container to roof level and spreading material. |  |
| Hazards | Site activity and conditions; Access; Handling hot bitumen, falling persons/materials, LPG |  |
| Risks | Falls from height (persons/materials); Skin contact (burns), Fire/explosion. |  |
| Population Exposed | Operative primarily. |  |
| Controls Statutory | Scaffolds/Working platforms where used, inspected and entry in Register. Edge protection required. Hoist to comply – competently erected, hoist guarded, competently used and checked each day of use and examined every 6 months. If Gin Wheel used, safety hook to be used. Head protection (working below). |  |
| Other Controls | Bitumen boiler set up in safe and secure area. LPG spare containers stored securely away from boiler. Material transferred into suitable container (not overfilled), and transferred to hoistway and lifted to roof level. Fire extinguishers to be readily available to boiler. Access arrangements to statutory standards and checked daily for adequacy. Working area – including boiler and access to below roof edge, securely fenced/screened off to protect others. Hot bitumen – low toxicity material selected, avoid contact (burns), good personal hygiene, provision of hand cleaning material. (COSHH Assess) |  |
| Consultation | Above to be discussed at pre-work meeting with employees and occupier (where applicable) |  |
| Documentation | Standard statement of safe working with hot bitumen for roof work. |  |
| Implementation | Supervisor to ensure boiler safety and access arrangements satisfactory and area fenced/screened off. Supervisor/Operative to check that access to working areas is safe. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks/precautions for hot bitumen (burns), roof work (access) |  |
| Training | Ensure operatives receive training in use of hot bitumen boiler, roof work and use of hoist (minimum one competent person for hoist). Use of extinguishers. |  |
| PPE | Head and Foot protection. May require protective clothing to protect against bitumen splashing/burns. |  |
| Monitoring | Constant supervision to check arrangements during progress of job. |  |
| Systems Review | Review periodically to confirm validity of safety arrangements. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref 21 | Plant Operators - Dumpers | MEDIUM RISK |
| --- | --- | --- |
| Method | Driving dumper trucks around site collecting and delivering materials. |  |
| Hazards | Site activity and conditions; Noise, Fuel/Lubricants, Materials, Head and Foot |  |
| Risks | Vehicle accident, noise induced hearing loss, skin exposure, Head and Foot. |  |
| Population Exposed | Operative and anyone on site. |  |
| Controls Statutory | Prohibition on all but trained, competent persons 18 years or older (unless under instruction by qualified persons). Carrying passengers is prohibited. Vehicle used regularly on public highways are subject to normal controls including road fund licence etc. |  |
| Other Controls | Drivers to be in possession of written authorisation to drive dumpers, received only if formally trained and passed proficiency test. If truck to be used in tight area, trained banksman to assist vehicle driver in manoeuvring. Where possible, traffic routes around/across site to be designated and clearly marked with right of way (vehicles or pedestrians). Vehicles subject to routine maintenance in accordance with manufacturer’s instructions. Vehicle engines to be switched off when not in use. Handling diesel/lubricants………… (COSHH Assess). Feet – site is foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at pre-work meeting with employees. |  |
| Documentation | Written policy on training and competence for authorisation. Disciplinary offence to drive without authorisation or ride as passenger/take passenger on vehicle. |  |
| Implementation | Supervisor to ensure only authorised persons drive dumpers. Driver to insect vehicle each day of use, and regular full inspection/maintenance by competent person. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks/precautions for driving dumpers. |  |
| Training | Ensure operatives receive formal training before authorised to drive (note: to drive dumper on public road requires full driving licence). Drivers also trained to carry out simple pre-use checks on vehicle. And to report defects including noisy exhaust etc. To correctly stow loads for safe carriage. CITB CTA Scheme or similar required. |  |
| PPE | Head protection (site work) and safety Footwear to be worn. May require hand protection, hearing defenders (subject to noise assessment). G21 continued………… |  |
| Monitoring | Supervisor to check that only authorised drivers use trucks, and no passengers, load carefully stacked etc. |  |
| Systems Review | Review periodically to confirm validity of Method Statement. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref 22 | Plant Operators – Excavators and cranes | HIGH RISK |
| --- | --- | --- |
| Method | Operating a variety of items of plant, for slinging loads, excavating etc. |  |
| Hazards | Site activity and conditions; Noise, Fuel/lubricants, Material, Head and Foot, Handling materials (hand). |  |
| Risks | Vehicle accident/overturning, noise induced hearing loss, skin exposures, Head and Foot, Handling materials (hand injury). |  |
| Population Exposed | Operative and anyone on site. |  |
| Controls Statutory | Prohibition on all but trained, competent persons 18 years or older (unless under instruction by qualified persons). Cranes are subject to test and examination 4 yearly (form 96) and examination weekly (form 91). Excavators are subject to test and examination 14 monthly and examination weekly (form91). |  |
| Other Controls | Drivers to be in possession of written authorisation to operate equipment, received only if formally trained and passed proficiency test. If plant to be used in tight area, trained banksman to assist driver in manoeuvring/slinging load. Where possible traffic routes around/across site to be designated and clearly marked with right of way, Plant subject to routine maintenance in accordance with manufacturer’s instructions. Vehicle engines to be switched off when not in use. All lifting operations to be carefully planned and safe system of work developed. An appointed person should evaluate the work and select suitable equipment. Work positions should be established and clearly set out. Feet – site is foot protection zone, safety boots to be worn. Handling diesel/lubricant…………….. (COSHH Assess) |  |
| Consultation | Above to be discussed at pre-work meeting with employees. |  |
| Documentation | Formal written procedures required: appointing persons; defining work; identifying positions of equipment; specifying methods and precautions. |  |
| Implementation | Supervisor to only ensure that competent persons are in control of lifting operations and use of other plant. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks/precautions for each item of plant to be used. |  |
| Training | Ensure operatives receive formal training before authorised to operate plant/cranes. Drivers also trained to carry out simple pre-use checks on plant, and to report defects. To correctly sling loads etc. Training to include for correct positioning of plant, such as excavators, adjacent to excavations, and safe in excavating. CITB CTA Scheme or similar required. |  |
| PPE | Head protection (site work), Foot protection. May require hand protection. G22 continued…………. |  |
| Monitoring | Supervisor to check that only authorised drivers use plant, and do so in accordance with written plan for work. |  |
| Systems Review | Review periodically to confirm validity of written plans, competency of authorised persons etc. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref 23 | Demolition – Soft (non-Structural | LOW RISK |
| --- | --- | --- |
| Method | Demolition of non-structural building elements including partitions, suspended ceilings, internal cladding, plaster etc (asbestos separate assessment) |  |
| Hazards | Site activity and conditions; Access; Dust, exposure to service, manual handling (waste), Noise, vibration, head and foot, hidden services. |  |
| Risks | Falls from height; dust inhalation, eye injury, skin contact; electric shock, gas leaks, back strain injury, hearing loss, vibration white finger, Head and Foot injury. |  |
| Population Exposed | Operative primarily |  |
| Controls Statutory | Access – requirements apply to demolition work. Working platforms, ladders etc, should be to the same standard as for other construction processes. Equipment such as hoists and cranes required statutory inspections and operation by competent persons. |  |
| Other Controls | Work plan required to provide reasonable access arrangements, ensure services are cut/isolated to area subject to demolition works and locked off, waste routes to be minimised by use of mechanical aids. Area to be surveyed to identify hidden services, asbestos or other toxic materials before work commences. Doubtful materials to be left undamaged until identified. Noisy equipment to be minimised and subject to noise assessment to identify requirements for hearing defenders. Vibrating tools to be selected for insulated handles, and gloves supplied for cold weather. Dust to be minimised and dust masks used, good personal hygiene, gloves used where appropriate. Feet – site is foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at pre-work meeting with employees. |  |
| Documentation | Outline Method Statements and COSH Assessments required. |  |
| Implementation | Supervisor/Operatives to check that access to working areas is safe, ladders are suitable, etc. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks/precautions for demolition work, instructed in correct use of tools and equipment. Instructed to leave suspicious materials undamaged and report them for investigation (asbestos, toxic residues etc). |  |
| Training | Training in good working practices and arrangements described in Method Statement…kinetic handling briefing for handling waste materials. G23 continued……………. |  |
| PPE | Hard hats and safety boots required. May require dust masks (typically for nuisance dusts), gloves, hearing defenders(subject to specific assessment) |  |
| Monitoring | Supervisor to occasionally check arrangements during progress of job. May require specific monitoring for dust, noise etc, subject to be employed and nature of site. |  |
| Systems Review | Review periodically to confirm validity of standard approach to works. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref 24 | Demolition – General demolition of structures | HIGH RISK |
| --- | --- | --- |
| Method | Demolition of structure by use of mechanical devises and equipment. |  |
| Hazards | Site activity and conditions; Access; Collapse of structures, dust, exposure to services, manual handling (waste), noise, vibration, machinery, structural stability (pre-stressed concrete). Head and Foot. |  |
| Risks | Falls from height; crushing by collapsed structures, dust inhalation, back strain injury, hearing loss, vibration white finger, Head and Foot injury. |  |
| Population Exposed | Operative primarily and others in vicinity including adjacent premises. |  |
| Controls Statutory | Demolition requires notification to local authority. Access – requirements apply to demolition work. Working platforms, ladders etc to the same standards as for other construction processes. Equipment such as hoist and cranes require statutory inspections and operation by competent persons. |  |
| Other Controls | Demolition to be preceded by full survey, characterising type of structure, conditions, need for temporary works or shoring, confined spaces, overhead services, hazards such as asbestos, lead paint on steelworks or other toxic residues. Minimise working at heights, removal/making safe of gas/electric/other services, method for dealing with toxic/flammable residues, waste transport and disposal, safety co-ordination. Minimise noise, and noise assessment to select hearing defenders. Vibrating tools to be selected for insulated handles, gloves supplied for cold weather. Feet – site is a foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at pre-work meeting with employees. |  |
| Documentation | Detailed Method Statement (sequence and method and COSHH Assessments) |  |
| Implementation | Safety co-ordination and supervision essential to ensure safe working practices employed and detailed arrangements in Method Statements implemented. Safe distances established for each phase of work to be monitored. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks/precautions for demolition work, instructed in correct use of tools and equipment. Instructed to leave suspicious materials undamaged and report them for investigation. |  |
| Training | Supervisors require training in their statutory responsibilities, project planning, establishment and running of site organisation. Operative training in good working practices during demolition. Kinetic handling briefing for operatives. G24 continued…………. |  |
| PPE | Hard hats and safety boots. May require dust masks (typically for nuisance dusts), gloves, hearing defenders (subject to specific assessment). If lead or asbestos present require formal assessment and selection of suitable PPE. |  |
| Monitoring | Safety Co-coordinator/supervisor to constantly check arrangements. May require specific monitoring for dust, noise, lead, etc, subject to work methods and nature of site. Health surveillance for operatives may be required; mandatory is exposed to significant lead (as dust or fumes from hot work). |  |
| Systems Review | Review periodically to confirm validity of Method Statement for works. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref 25 | Steel Erection | HIGH RISK |
| --- | --- | --- |
| Method | Variety of tasks associated with the erection of steel framework as a phase in the erection of the structure. |  |
| Hazards | Site activity and conditions; Falls from height, head, foot and handling items. |  |
| Risks | Falls from height, Head, Foot, Hand injury. |  |
| Population Exposed | Operative primarily. |  |
| Controls Statutory | Access – requirements apply to steel erection. Working platforms, ladders etc, should be to the same standard as for other construction processes. Equipment such as hoists and cranes require statutory inspections and operation by competent persons. |  |
| Other Controls | Survey to include for overhead power lines, access to work area, etc. The works should be carried out to a design plan which includes the detailed sequence for erection. The erection scheme to be detailed in a Method Statement with annotated diagrams, following a logical sequence. Materials received to site should be stored for stability; protection of nibs and brackets and ease of De-stacking. Fabrication at ground level maximised, to limit working at height use of elevating platforms and buckets to be considered for safe access. Access along beams above 2m minimised, securely guarded or use of running line or anchorage for harness for top beam(unless over 0.8m wide and fitted with continuous handhold). Beam straddling acceptable, but fall protection to working position required. Feet – site is foot protection zone, safety boots to be worn. |  |
| Consultation | Above to be discussed at a detailed pre-work meeting with employees. |  |
| Documentation | Method Statement with detailed sequence and annotated drawings. |  |
| Implementation | Work to be directly managed by competent person, implementing agreed and documented Method statement. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on Method Statement before work permitted. |  |
| Training | Supervisor/competent person requires training on his statutory responsibilities, project planning, establishment and running of package. Ensure operatives receive basic training on safety in steel erection. |  |
| PPE | Hard hats and safety boots. Safety nets and belts/harnesses should be used as required. |  |
| Monitoring | Constant supervision to regularly check arrangements during progress of job. |  |
| Systems Review | Review regularly to confirm validity of Method Statement. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G26 | Scaffolding – Mobile towers | LOW RISK |
| --- | --- | --- |
| Method | Erection and use of proprietary mobile tower. |  |
| Hazards | Site activity and conditions; Flooring, overhead structures/services, head and foot. |  |
| Risks | Falls from height; contact with overhead structures/services, head and foot injury. |  |
| Population Exposed | Operative primarily. Adjacent other operatives and others. |  |
| Controls Statutory | Regulations for scaffolds apply to mobile towers – safe, stable, sturdy, toe boards, guard rails, access ladders etc. PASMA Code of practice to be adopted. |  |
| Other Controls | Towers to be erected by authorised persons trained in work (under direct supervision), and in possession of supplier’s handbook. Tower to be erected and used in areas where floor is free of obstructions, holes etc, and where height does not represent risk of contact with overhead structures – unless these are noted and carefully avoided during use. Towers to be erected with appropriate outriggers to ensure height to base ratio is maintained, guard rails, toe boards, ladders etc. Not to be moved when men or materials on platforms. Platform to be inspected before each use. |  |
| Consultation | Above to be discussed at pre-work meeting with employees. During erection and dismantling access to area to be restricted – requires action by client/occupiers/main contractor. |  |
| Documentation | Supplier’s Handbook. |  |
| Implementation | Competent person to ensure erected appropriately. Operatives to check that access to platform is safe, ladders are suitable. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks/precautions for use of tower scaffolds. |  |
| Training | Competent person to supervise/carry out erection to receive formal training. |  |
| PPE | Hard hats and safety boots may be required. |  |
| Monitoring | Competent person and or operatives to visually check tower each day, and after each relocation. |  |
| Systems Review | Review periodically to confirm validity of safe system of work |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G27 | Scaffolding – Independent Tied Scaffold | MEDIUM RISK |
| --- | --- | --- |
| Method | Erection/dismantling of standard scaffold, independent of building but tied to it for stability. |  |
| Hazards | Falls from height, Head and Foot, Noise, Portable tools, overhead power lines. |  |
| Risks | Falls from height, Head and Foot injury, Hearing loss and electric shock. |  |
| Population Exposed | Operative and others working/passing below. |  |
| Controls Statutory | Scaffold standards (minimal) specified in Regulations. Inspections and entry in Register. Noise assessments for power tools. Electric tools to be run on low voltage, inspected. |  |
| Other Controls | Erection/dismantling by competent, trained and experienced persons or under competent direct control and carried out in accordance with statutory requirements and code of practice, to satisfy designed criteria (load, usage etc) CITB Scaffold registration scheme provides mechanism for establishing competency of operations. |  |
| Consultation | Above to be discussed at pre-work meeting with employees. During erection and dismantling access to area to be restricted – requires action by client/occupier/main contractor. |  |
| Documentation | Purpose of scaffold, as identified to designer, should be written down. This to include working loads of men and materials, which should not be exceeded. |  |
| Implementation | Scaffolding supervisor to ensure that scaffold is erected in accordance with design. During work, safety of operatives depends on following good working practice, remaining in secure positions as far as possible and taking personal care. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks of working receive kinetic handling briefing. |  |
| Training | Supervisor and operatives to be registered under CITB Scheme. Ensure operatives receive kinetic handling briefing. |  |
| PPE | Scaffolder’s hard hat, gloves and foot protection (mid-sole protection not advisable for scaffolding). (Note; When working within standards on a three board intermediate platform, safety harnesses are not normally required as they may increase the risk of tripping/falling). Hearing defenders if noise assessment indicates requirement. |  |
| Monitoring | Supervisor to periodically check arrangements during progress of job. |  |
| Systems Review | Review periodically to confirm validity of designs and training methods. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G28 | Scaffolding – High slung, Trussed out Or Cantilevered | HIGH RISK |
| --- | --- | --- |
| Method | Erection/dismantling of scaffolds dependent on building/structure for support and stability. |  |
| Hazards | Falls from height, Head and Foot, Noise, Portable tools. |  |
| Risks | Falls from height, Head and Foot injury, Hearing loss, electric shock. |  |
| Population Exposed | Operative and others working/passing below. |  |
| Controls Statutory | Scaffold standards (minimal) specified in Regulations. Inspections and entry in Register. Noise assessments for power tools. Electric tools for fixing ring insertion to be run on low voltage, inspected. |  |
| Other Controls | Design to take into account stability of existing structure and detailed precautions required to ensure maximum safety. Erection/dismantling by competent, trained and experienced persons or under competent direct control and carried out in accordance with statutory requirements and code of practice, to satisfy designed criteria (load, usage etc). Individual components (tubes, couplers, etc) to be to British Standards and inspected before use. |  |
| Consultation | Above to be discussed at pre-work meeting with employees. During erection and dismantling access to area to be restricted – requires action by client/occupier/main contractor. |  |
| Documentation | Purpose of scaffold, as identified to designer, should be written down. This to include working loads of men and materials, which should not be exceeded. Detailed design required. |  |
| Implementation | Scaffolding supervisor to ensure that scaffold is erected in accordance with design. During work, safety of operatives depends on following good working practice, remaining in secure positions as far as possible and taking personal care. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Briefing on risks of working off incomplete scaffolding. |  |
| Training | Supervisor and operatives to be registered under CITB Scheme. Ensure operatives receive kinetic handling briefing. |  |
| PPE | Scaffolder’s hard hat, gloves and foot protection (mid-sole protection not advisable for scaffolding). Suitable fall arrest equipment required. Hearing defenders if noise assessment indicates requirements. |  |
| Monitoring | Constant supervision to regularly check arrangements during progress of job. |  |
| Systems Review | Review periodically to confirm validity of designs and training methods. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G29 | Stone Mason – Stone Cleaning | HIGH RISK |
| --- | --- | --- |
| Method | Cleaning stonework with hydrofluoric acid. |  |
| Hazards | Acid burns, falls from height (access)** |  |
| Risks | Serious damage to skin and eyes, acid highly corrosive may penetrate to deep tissue. Subject to position – may be serious risk of falls. Contamination of scaffold boards and tubing. |  |
| Population Exposed | Operative primarily, others working adjacent to or below potentially exposed to spillage/spray of acid. Occupier and public – access adjacent to or below working area. |  |
| Controls Statutory | Scaffold/Working Platforms/Ladder standards and other aspects of safe access. Head protection. Foot, face, hands and body protection. |  |
| Other Controls | Method Statements defining safe storage, transport and use of acid (COSHH Assess) Emphasis on containment, and within containment protection of operatives. Cleaning of access equipment. First Aid – availability neutralising gel (calcium gluconate) and trained first aider. |  |
| Consultation | Above to be discussed at pre-work meeting with employees and occupiers. |  |
| Documentation | Written Method Statement required for safe use of hydrofluoric acid. |  |
| Implementation | Supervisor to check at commencement that above arrangements in force. |  |
| Information Instruction | New employees – induction on company policy and procedures and for safe working before permitted to work. Specific instructions on safe working with acid. |  |
| Training | Ensure operatives receive specific training on safe handling of acid First aider trained in treatment of acid burns. |  |
| PPE | Hard Hat; Impervious Gloves, overalls and safety footwear, face visor. |  |
| Monitoring | Constant supervision to recheck arrangements during progress of job. Containment of acid to be checked. |  |
| Systems Review | Overall review of safety at conclusion of each job. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G30 | Asbestos Materials | HIGH RISK |
| --- | --- | --- |
| Method | Demolition of non structural building elements including partitions, cladding, plaster coating, Artex, flue liners and ventilation ducting. |  |
| Hazards | Site activity and conditions; Access; dust, manual handling of contaminated materials, inhalation of fibres. |  |
| Risks | Dust inhalation, Asbestos. |  |
| Population Exposed | Operatives and possibly the general public. |  |
| Controls Statutory | This material is covered by the Asbestos regulations and the current ACOP on asbestos. |  |
| Other Controls | Risk assessments should be carried out on all suspected materials, with special attention to fibre boards, sheets, cladding, flue pipes, ventilation systems and boiler systems. If a suspect material is found then adequate PPE should be worn. DO NOT TOUCH THE MATERIAL CALL FOR EXPERT ASSISSTANCE! Close the premises or area down. Notify the office. If contaminated then all clothing and PPE should be places into a sealable plastic bag. All personnel should shower and put on clean clothes (paper overalls in an emergency). |  |
| Consultation | Operatives should have been informed of policy on asbestos materials and enquiries made for asbestos register. |  |
| Documentation | Method Statements are enquired from licensed operators if they have to work while other operatives are on site. All areas will be nominated with Asbestos notices. |  |
| Implementation | Site Agents and Supervisors to ensure area is secured and that a licensed operator is called. |  |
| Information Instruction | New employees – induction on company policy and procedures and what to do if asbestos is suspected. Other employees – refresher talks and toolbox talks. |  |
| Training | Carried out in house and by external instructors. |  |
| PPE | PPE is to be worn at all times as required. This matter is the sole responsibility of the specialist operator. |  |
| Monitoring | If a person is contaminated then action will be taken to ensure that periodic checks are carried out. The information will be placed on the individual’s record card held at the main office. |  |
| Systems Review | This assessment shall be reviewed annually, after incidents and if there is a change in the legislation. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G31 | Cement Mixing – Diesel/Petrol mixer | MEDIUM RISK |
| --- | --- | --- |
| Method | Mixing cement and mortar by use of mechanical cement mixer, petrol or diesel driven engine. |  |
| Hazards | Site activity and conditions, Moving traffic, Loading out, dust and wet motar. Burns due to cement dusts, broken/damaged thumb/leg from misuse of starting handle. |  |
| Risks | Back strain, breaking of bones in the hand and knee by misuse of starting procedures. Special care should be taken when first starting the machine with crank handle. Respiratory problems due to dust, eyes and skin. Foot, knee and hand injuries. |  |
| Population Exposed | Operative primarily, fellow workers in the area. |  |
| Controls Statutory | Health & Safety at Work Act, PUWER, Management Regulations. PPE Regulations and Manual Handling Regulations. |  |
| Other Controls | Dust – work upwind Skin – overalls, gloves, boots, hand washing facilities. Mechanical – check daily by operative all faults reported to Site Agent. Manual Handling – use barrows for moving materials never carry motar in buckets up ladders. Operatives to be familiar with kinetic lifting methods. Starting of machine with thumb and fingers on same side of handle. |  |
| Consultation | All above to be discussed and operator to be shown operation methods at induction. |  |
| Documentation | COSHH Assessment contained on all materials i.e. cement bags. |  |
| Implementation | Site Agent to oversee and carry out induction of new employees and ensure that they are familiar with the equipment they are going to use. |  |
| Information Instruction | Induction, Toolbox talk, Method Statements discussed when appropriate. |  |
| Training | Ensure operatives have been trained and are competent. |  |
| PPE | Boots, hat, Gloves, Overalls, Hearing protection if required and Goggles. |  |
| Monitoring | Site Agent to recheck that the operator is working safely and within company policy. |  |
| Systems Review | Not normally carried out on this individual task. |  |

WORK ACTIVITY ASSESSMENT

| Work Activity Ref G32 | Working at height | HIGH RISK |
| --- | --- | --- |
| Method | All systems requiring an operator to work at height, from which he is likely to injure himself in a fall. |  |
| Hazards | Operatives falling from height. Persons walking/working beneath. |  |
| Risks | Serious injury from persons falling or materials debris falling from height. |  |
| Population Exposed | Operatives, visitors, pedestrians. |  |
| Controls Statutory | All visitors to site to be signed in and inducted. Use a safe means of access and egress from work place. Edge protection to be provided to all areas where any fall may occur where an injury is likely. Working platform to be adequately protected to prevent the fall of debris or material. |  |
| Other Controls | Entrances to buildings scaffold over pathways to be close boarded or ply sheets in place to prevent fall of persons and debris on pedestrians. |  |
| Documentation | Method statement to be in place prior to work commencing. Sub Contractors must also comply if they are involved in this type of work. Method statement to be signed by all parties involved. |  |
| Implementation | Site agent/Supervisor to ensure that only competent persons are working at height or under the direct supervision of an experienced operative. |  |
| Information Instruction | New employees will be inducted to the site and made aware of the site rules, and company policy and procedures for working at height. All safety precautions including P.P.E and the use of harness etc will be detailed and operator trained in their use. |  |
| Training | All operatives to receive formal training and ensure the sub contractors have received or are experienced in this type of work prior to the work commencing. |  |
| PPE | Head protection, toe tector boots must be worn, other P.P.E as dictated by the work and site agent will be worn (i.e. Harness, netting etc) |  |
| Monitoring | The site agent and independent advisors will check the area regularly to ensure compliance with legislation. |  |
| Systems Review | This procedure will be reviewed on an annual basis or as is found to be necessary. |  |